School of Science and Technology

# Electronic Devices and Materials Technology ENGG10031

Academic Year 2022-2023

Lab 8: MOSFET – Characteristics and applications

# Objectives

- Studying transfer and output characteristics of MOSFETS

- Using MOSFET as a switch

# Equipment and components:

- MOSFET 2N7000

- Breadboard

- resistor (1kΩ, 100kΩ, 560Ω)

- DC power supply

- Digital mustimeters (DMM) × 3

- LED (L-53YD)

- Signal generator

- connecting wires

# Required information

## Introduction to the MOSFET

The MOS(metal-oxide-semiconductor) transistor (or MOSFET) is the basic building block of most computer chips, as well as of chips that include analog and digital circuits. In this lab, you will work with what is called an n-channel MOS transistor.

A common symbol for the N-channel MOS transistor is shown in Figure 1(a). Of the terminals shown, the ones we will focus on are the source, drain, and the gate. In the transistor we will be working with in this lab, the fourth terminal, labelled body in Figure 1(a), is not accessible externally; rather, it is internally connected to the source, as shown by the broken line. For simplicity, we will use the symbol shown in Figure 1(b).

(a)                                                                                     (b)

Figure 1 MOSFET

To study the MOSFET, two external voltage sources are required, as shown in Figure 2. These provide the drain-source voltage VDS and the gate-source voltage VGS. The voltage VDS may cause a drain-to-source current iD as shown, provided that there is a path for this current from the drain through the device to the source. Whether such a path exists or not depends on the value of VGS. For small values of VGS, no such path is established within the device, and iDS is zero. The device then looks like an open circuit between the drain and the source. For a sufficiently high VGS, an internal current path, called a channel, is established between the drain and the source. Now a current can flow. The precise value of VGS determines how easy it is for the channel to conduct the current; the higher the value VGS, the easier such conduction is and the larger the value of iDS. For a given VGS, the value of iDS will also depend on VDS and will tend to increase with the latter.

## Transfer and output characteristics of a MOSFET

In this experiment, you will observe the transfer and output characteristics of an NMOS transistor. Transfer characteristics features the dependency of the drain current on gate voltage. The simplified reference circuit for this experiment and the expected response is shown in Figure 2.

Figure 2. The reference circuit and transfer characteristics of a MOSFET

The output characteristics illustrate the dependency of the drain current on drain voltage. Since this depends on the gate voltage, we can obtain a family of graphs. Figure 3 shows the simplified reference circuit for this experiment and the expected family of curves.

Figure 3. The reference circuit and output characteristics of a MOSFET

## Definitions of terms:

  - Threshold voltage VT: Gate to source voltage at which, drain current starts flowing.

  - Transconductance gm: Ratio of small change in drain current (ΔID) to the corresponding change in gate to source voltage (ΔVGS) for a constant VDS.

  - gm = ΔID/ΔVGS at constant VDS

  - Output drain resistance: It is given by the ratio of small change in drain to source voltage (ΔVDS) to the corresponding change in drain current (ΔID) for a constant VGS.

  - rd or ro = ΔVDS / ΔID at a constant VGS

## MOSFET as a switch

When a MOSFET is operated under cut-off regions (vGS≤Vt), it can block the drain current due to very high resistance between the drain and source. On the other hand, when it is operated in the triode region (vDS≤VOV), the conductivity between gate and source becomes high due to the formation of induced channel. Hence, MOSFET facilitates the current flow and impose only a little voltage drop. Therefore, the aforementioned two states of MOSFET transistors are perfect for switching applications since the power loss within the transistor is minimal. The CMOS family of logic devices utilises MOSFET’s excellent switching capability.

(a)	(b)

Figure 5. MOSFET operates under (a) cut-off and (b) triode states

## MOSFET datasheet

The MOSFET datasheet can be downloaded from the now learning room (lab 8 MOSFET). They can also be found at the following websites:

https://www.onsemi.com/pdf/datasheet/nds7002a-d.pdf

# Procedures:

## Output characteristics:

- Connect the circuit as per given diagram shown in Figure 4. Utilize channel 1 of the DC power supply to provide voltage VGG and channel 2 of the DC power supply to provide voltage VDD. Utilize the DMM to measure the drain current by series connection with the resistor RD and drain terminal (positive lead connects to the resistor, negative lead connects to the drain terminal). Use the white “3A” port and right black port for the current measurement. Click on “SHIFT” and then “DCV” to set the DMM to measure current.

- Utilise the channel 1 of the oscilloscope to measure the gate-source voltage (VGS) and channel 2 of the oscilloscope to measure the drain-source voltage (VDS).  Note that you will need to adjust the scale of the voltage measurement to be corresponding to the voltage values to be measured (i.e. slightly bigger). The DC voltage can be read by click “Measure”, and then choose “Mean”.

- Keep VGS constant at a value of 1.1 V by varying VGG.

- Vary VDS in step of 1V up to 10 volts and measure the drain current ID. Tabulate all the readings in Table 1 in the results section.

- Repeat the above procedure for VGS as 1.3V, 1.5V.

- Plot the output characteristics VDS vs ID. The horizontal axis should be VDS and the vertical axis ID. There should be one curve for each VGS, namely, the various VGS values generate a family of curves. You should be able to observe on the plot that, for each curve, the current tends to a constant (or, as we say, saturates) as VDS is made large.

Figure 4. Circuit diagram

## Transfer characteristics:

- Using the same circuit as above, set the voltage VDS constant at 10 V.

- Vary VGS by varying VGG in the step of 0.1 up to 1.55V and note down value of drain current ID. Tabulate all the readings in Table 2.

- Plot the transfer characteristics VGS vs ID. The horizontal axis should be VGS and the vertical axis ID.

- Calculate VT and gm from the transfer characteristics curve obtained, and calculate ro from the output characteristics curve obtained, verify it from the data sheet.

## MOSFET as a switch (Optional):

- Connect the circuit as per given diagram shown in Figure 5. Use a signal generator to provide voltage to the gate.

- Set the signal generator so that It outputs 3.3V square wave with a frequency of 1Hz.

- Observe the response of the LED. Comment on the function this circuit is performing.

Figure 5 Switch circuit

# Results:

Table 1 Output characteristics

| VGS=1.1V |  | VGS=1.3V |  | VGS=1.5V |  |
| --- | --- | --- | --- | --- | --- |
| VDS (V) | ID (mA) | VDS (V) | ID (mA) | VDS (V) | ID (mA) |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

Table 2 Transfer characteristics

| VDS=10V |  |
| --- | --- |
| VGS (V) | ID (mA) |
| 0 |  |
| 0.1 |  |
| 0.2 |  |
| 0.3 |  |
| 0.4 |  |
| 0.5 |  |
| 0.6 |  |
| 0.7 |  |
| 0.8 |  |
| 0.9 |  |
| 1.0 |  |
| 1.1 |  |
| 1.2 |  |
| 1.3 |  |
| 1.4 |  |
| 1.5 |  |
| 1.55 |  |

- Vt:                                       .

- gm:                                       .

- ro:                                       .

Comment on the function the circuit in Figure 5 is performing:

# Post lab Questions:

- To turn on the NMOS, how much voltage is required?

- Why an input characteristic of MOSFET is not drawn?

Answer: Input characteristic means IG vs VGS. However due to the high input impedance of the MOSFET, IG=0.